*** NVIDIA Jetson AI Lab ***

- AI Microservices – Introduction:

- Run and Enable the microservice in the jeson nano machine || run API in locan network

- https://www.jetson-ai-lab.com/models.html

- The Microservices is start the API in NVIDIA jetson nano orin Developer kit

- Copy Docker Container Command

- Start docker Container by this command

- docker run -it -p 9000:9000 dustynv/ollama:main-r36.4.0

- Ip Table Row table error (iptables_row) then run this:  (imp)

  - docker run -it --network host dustynv/ollama:main-r36.4.0

- After the Container is Start Pull the LLAMA Module

- ollama run gemma3:1b

- ollama run gemma3:4b

- ollama run gemma3:12b

- etc

- this is the API End-Point

- http://192.168.1.111:11434/v1/chat/completions

- Run Open-WebUI:

docker run -it --rm \

--name open-webui \

--network=host \

-e PORT=8080 \

-e ENABLE_OPENAI_API=True \

-e ENABLE_OLLAMA_API=False \

-e OPENAI_API_BASE_URL=http://192.168.1.111:11434/v1 \

-e OPENAI_API_KEY=foo \

-e AUDIO_STT_ENGINE=openai \

-e AUDIO_TTS_ENGINE=openai \

-e AUDIO_STT_OPENAI_API_BASE_URL=http://192.168.1.111:8990/v1 \

-e AUDIO_TTS_OPENAI_API_BASE_URL=http://192.168.1.111:8995/v1 \

-v /mnt/nvme/cache/open-webui:/app/backend/data \

-e DOCKER_PULL=always --pull always \

-e HF_HUB_CACHE=/root/.cache/huggingface \

-v /mnt/nvme/cache:/root/.cache \

ghcr.io/open-webui/open-webui:main

start the front-end to call that microservices from front-end

- Run this Docker command in the terminal (192.168.1.111:11434 -> replace)

- Llama Chat in Local Network:

- Doc: https://my.cytron.io/tutorial/jetson-orin-nano-google-gemma-2-personal-assistant&tracking=yt-products&utm_campaign=yt-products

- Youtube: https://www.youtube.com/watch?v=2VljMU26Zq4&list=WL

- Library

- Jtop: jtop is a system monitoring tool providing real-time Jetson performance information. It displays information about the usage of the CPU, GPU, memory and other system resources. jtop displays the output from the Jetson power monitors and temperature sensors.

- Jetson Container:  A container is an executable unit of software where an application and its run time dependencies can all be packaged together into one entity. Since everything needed by the application is packaged with the application itself, containers provide a degree of isolation from the host and make it easy to deploy and install the application without having to worry about the host environment and application dependencies.

Possible Tasks for Company Automation with Jetson Orin Nano Developer Kit

1. Robotics and Autonomous Systems

- Autonomous Navigation Robots: Develop robots for warehouse automation, such as automated guided vehicles (AGVs) or autonomous mobile robots (AMRs). Use NVIDIA Isaac ROS for navigation, path planning, and obstacle avoidance, leveraging the kit’s vision processing capabilities with MIPI CSI camera connectors.

- Robotic Arms for Pick-and-Place: Implement AI-powered robotic arms for tasks like sorting, packing, or assembly in manufacturing. Use vision transformers (ViTs) for object detection and grasping, supported by Hugging Face LeRobot for action prediction from visual inputs.

- Inspection Robots: Create robots for quality control, such as inspecting products on a production line using cameras and vision-language models (VLMs) for defect detection or classification.

- Service Robots: Build robots for customer service or facility management, like delivery robots in offices or hospitals, using large language models (LLMs) for interaction and DeepStream for real-time video analytics.

2. Computer Vision and Video Analytics

- Smart Surveillance Systems: Deploy AI-powered security cameras for real-time monitoring, facial recognition, or anomaly detection in workplaces using NVIDIA DeepStream and pretrained models from the NGC catalog (e.g., PeopleNet for person detection).

- Object Detection and Tracking: Implement systems to track inventory, tools, or equipment in warehouses or factories. Use vision transformers for image classification or object detection to automate inventory management.

- Quality Inspection: Automate visual inspection of products for defects or compliance using the Vision Programming Interface (VPI) and pretrained models fine-tuned with NVIDIA TAO Toolkit.

- Smart Retail Analytics: Develop systems to monitor customer behavior, track foot traffic, or manage shelves in retail environments using NVIDIA Metropolis for video analytics.

3. Generative AI Applications

- AI Chatbots for Automation: Build generative AI chatbots using LLMs like Llama-3.1-8B or Ollama with Open WebUI for internal company tasks, such as automating customer support, HR queries, or technical assistance. These can run locally on the Jetson Orin Nano for low-latency responses.

- Text-to-Image Generation: Create systems for generating visual assets or prototypes for product design using vision-language models (VLMs) or diffusion models, applicable in marketing or product development.

- Retrieval-Augmented Generation (RAG): Implement RAG-based systems for knowledge management, such as querying company documents or technical manuals with enhanced context-aware responses.

4. Industrial IoT and Smart Devices

- AI-Enhanced IoT Gateways: Develop intelligent gateways for collecting and processing data from IoT sensors in real-time, enabling predictive maintenance or equipment monitoring in factories. Use the kit’s GPIO, I2C, SPI, and UART interfaces for sensor integration.

- Environmental Monitoring: Create systems to monitor temperature, humidity, or air quality in facilities, using cameras or sensors connected via MIPI CSI or GPIO, with AI for anomaly detection.

- Energy Management Systems: Build smart energy monitors to optimize power usage in industrial settings, leveraging the kit’s low power consumption (7W–25W) and real-time data processing.

5. Factory Automation

- Real-Time Process Monitoring: Use DeepStream Graph Composer to create AI pipelines for monitoring production lines, detecting bottlenecks, or ensuring safety compliance with real-time video feeds.

- Predictive Maintenance: Implement AI models to analyze sensor data (vibration, temperature, etc.) and predict equipment failures, reducing downtime in manufacturing.

- Automated Sorting Systems: Develop vision-based sorting systems for materials or products on conveyor belts, using ViTs or pretrained models for classification and segmentation.

- Human-Robot Collaboration: Create systems for safe human-robot interaction in factories, using vision and language models to interpret human gestures or commands.

6. Smart City and Infrastructure Automation

- Traffic Management: Build systems for monitoring traffic flow or detecting violations at intersections using cameras and DeepStream for real-time video processing.

- Smart Parking Solutions: Develop AI-based parking management systems that detect available spaces or unauthorized vehicles using computer vision.

- Facility Access Control: Implement biometric or facial recognition systems for secure access to company premises, using the kit’s GPU for fast inference.

7. Healthcare Automation

- Medical Imaging Analysis: Use vision transformers for analyzing medical images (e.g., X-rays or scans) to assist in diagnostics or quality control of medical devices.

- Patient Monitoring Systems: Develop edge AI systems to monitor patient vitals or detect anomalies in real-time using sensors and cameras connected to the Jetson Orin Nano.

- Assistive Robots: Create robots for hospital logistics, such as delivering supplies or guiding patients, using NVIDIA Isaac for navigation and Riva for speech processing.

8. Logistics and Supply Chain Automation

- Inventory Management: Automate stock tracking and reordering using computer vision to monitor shelves or warehouses, integrated with IoT sensors.

- Drone-Based Delivery or Inspection: Develop AI-powered drones for inventory checks or last-mile delivery, using the kit’s vision and navigation capabilities.

- Supply Chain Analytics: Use LLMs or VLMs to analyze supply chain data, predict demand, or optimize logistics routes based on real-time inputs.

9. Custom Hardware Integration

- Custom Carrier Board Development: Design custom carrier boards for specific automation tasks, leveraging the Jetson Orin Nano’s compatibility with Orin NX modules and its detailed pinout documentation for GPIO, I2C, SPI, and camera connectors.

- Sensor Integration: Connect custom sensors (e.g., LiDAR, thermal cameras) via the kit’s MIPI CSI-2 or M.2 interfaces for specialized automation tasks like environmental mapping or thermal inspections.

- Edge-to-Cloud Integration: Build hybrid systems that process data locally on the Jetson Orin Nano and sync with cloud platforms for centralized monitoring or analytics.

10. Prototyping and Proof-of-Concept Development

- Rapid AI Prototyping: Use pretrained models from NVIDIA NGC or Hugging Face Transformers to quickly prototype automation solutions, fine-tuning them with the TAO Toolkit for company-specific datasets.

- Simulation and Testing: Leverage NVIDIA Isaac Sim for simulating robotic or automation workflows before deployment, ensuring robust performance in real-world scenarios.

- Educational and Training Tools: Develop in-house training platforms for employees to learn AI and robotics, using the Jetson Orin Nano as a hands-on learning tool.

Implementation Considerations

- Setup Requirements: Ensure the Jetson Orin Nano is updated to JetPack 6.1 for maximum performance (67 TOPS). Flash the microSD card with the latest image and consider adding an NVMe SSD for faster I/O and larger storage.

- Power Modes: Use the MAXN SUPER power mode (accessible via the NVIDIA icon on the Ubuntu desktop) to unlock full performance for compute-intensive tasks.

- Software Tools: Leverage NVIDIA’s software stack, including:

  - JetPack 6.1: Includes CUDA 12.6, TensorRT 10.3, VPI 3.2, and DLA 3.1 for AI and vision tasks.

  - NVIDIA Isaac: For robotics development.

  - DeepStream: For real-time video analytics.

  - Riva: For conversational AI.

  - Metropolis: For smart city and retail analytics.

- Hardware Integration: Utilize the kit’s rich I/O (4x USB 3.2, 2x M.2 Key M, M.2 Key E with Wi-Fi, MIPI CSI-2 camera connectors) for connecting cameras, sensors, and storage.

- Community Resources: Explore NVIDIA Jetson Forums, Jetson AI Lab, and community projects for tutorials and support.

Example Workflow for a Company Automation Project

- Identify Use Case: Choose a task, e.g., automating quality inspection in manufacturing.

- Hardware Setup: Connect cameras or sensors to the Jetson Orin Nano via MIPI CSI-2 or GPIO.

- Software Configuration: Flash JetPack 6.1, enable MAXN SUPER mode, and install necessary frameworks (e.g., DeepStream, TAO Toolkit).

- Model Development: Use pretrained models from NGC or Hugging Face, fine-tune with TAO Toolkit for your specific dataset.

- Deployment: Deploy the model on the Jetson Orin Nano for real-time inference, integrating with existing company systems.

- Testing and Optimization: Use Isaac Sim for simulation and jtop for monitoring performance.

- Scaling: Develop custom carrier boards or integrate with cloud systems for large-scale deployment.

Limitations and Notes

- Performance Constraints: While the Jetson Orin Nano Super offers 67 TOPS, it’s less powerful than higher-end Jetson models like the AGX Orin (275 TOPS), so prioritize tasks suited for edge deployment.

- Firmware Updates: Ensure the firmware is updated to support JetPack 6.x, as older firmware is incompatible.

- Cost-Effectiveness: At $249, the kit is affordable, but additional peripherals (cameras, sensors, NVMe drives) may increase costs.

- Learning Curve: Some tasks, like custom hardware integration or advanced model training, may require familiarity with Linux, CUDA, or ROS.