lo** IoT Code and Support **

- ESP32 GPIO PIN: ->

| GPIO | Special Function | Notes |
| --- | --- | --- |
| GPIO0 | Boot mode selection, LED control | Must be HIGH at boot to avoid flash mode |
| GPIO1 | UART TX (serial) | For serial communication (TX) |
| GPIO2 | Built-in LED, boot mode indicator | Must be LOW at boot for normal start |
| GPIO3 | UART RX (serial) | For serial communication (RX) |
| GPIO4 | General-purpose pin | Can be used for LEDs, sensors, etc. |
| GPIO5 | SPI CS (Chip Select) | Output capable, common for SPI. |
| GPIO12 | HSPI MISO (input). | Be careful — can affect boot process |
| GPIO13 | HSPI MOSI (output) | Common for SPI communication |
| GPIO14 | HSPI SCK (clock) | Used for SPI devices |
| GPIO15 | HSPI CS (chip select) | Must be pulled HIGH at boot |
| GPIO16 | RTC wake-up pin | Can wake ESP32 from deep sleep |
| GPIO17 | Standard GPIO | Safe for general use |
| GPIO21 | I2C SDA | I2C communication line |
| GPIO22 | I2C SCL | I2C communication clock |
| GPIO23 | SPI MOSI | Output for SPI. |
| GPIO25 | DAC Channel 1 | Can output analog voltage |
| GPIO26 | DAC Channel 2 | Can output analog voltage |
| GPIO27 | Touch sensor | Can detect touch events |
| GPIO32 | ADC (Analog input) | Read analog sensors |
| GPIO33 | ADC, Touch sensor | Can read analog values or touch. |

- Folder Structure:

Ans:- GitHub: https://github.com/embeddedartistry/project-skeleton

YouTube: https://www.youtube.com/watch?v=6oJ2LLxfP3s

- Important OS for Development

- Zephyr OS,

- Nucleus OS

- Big Companied Development Interface

| Use Case | Industry Standard Tools |
| --- | --- |
| ESP32 Development | ESP-IDF (Official SDK) |
| RTOS Development | FreeRTOS, Zephyr OS |
| Large-Scale Projects | CMake, Makefiles |
| Automotive & Aerospace | AUTOSAR, QNX, RTEMS |
| Industrial IoT Yocto | Embedded Linux |
| Advanced Debugging | JTAG, OpenOCD, Segger Ozone |

- MAC Address:->

- Master_Module -> ESP32-C3 : "84:FC:E6:00:B9:9C" -> {0x84, 0xFC, 0xE6, 0x00, 0xB9, 0x9C}

- Module_1 -> ESP32 : “94:54:C5:82:C4:48” -> {0x94, 0x54, 0xC5, 0x82, 0xC4, 0x48}

- Module_2 -> ESP32 : “C0:5D:89:B1:56:B8” -> {0xC0, 0x5D, 0x89, 0xB1, 0x56, 0xB8}

| ESP8266 | GPS | OLED | DHT | MPU9250/9500 | Button |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3V | GND | GND | GND | GND | (3V--10K) D7 |  |  |  |  |
| G | VCC | VCC | VCC | VCC or AD0 | GND |  |  |  |  |
| D0 | - | - | - | - | - |  |  |  |  |
| D1 | - | SCL | - | SCL | - |  |  |  |  |
| D2 | - | SDA | - | SDA | - |  |  |  |  |
| D3 | - | - | - | - |  |  |  |  |  |
| D4 | - | - | DATA | - |  |  |  |  |  |
| D5 | TX | - | - | - |  |  |  |  |  |
| D6 | RX | - | - | - |  |  |  |  |  |
| D7 | - | - | - | - | (D7--10K R) 3V |  |  |  |  |
| D8 | - | - | - | - |  |  |  |  |  |
| RX | - | - | - | - |  |  |  |  |  |
| TX | - | - | - | - |  |  |  |  |  |

- PULL-UP Button Connection :

3v ----> 10 k/omh ---> 1st side button // 2nd side button --> GND

D7 -->  10 k/omh 1st side button

Returning value is “1”

- PULL-DOWN Button Connection :

3v ----> 1st side button // 2nd side button --> 10 k/omh --->  GND

D7 -->  10 k/omh 2nd side button

Returning value is “0”

- LoRa (REYAX RYLR 890) - ESP32-C3 :->

| ESP32-C3 | LoRa (REYAX RYLR 890) |
| --- | --- |
| 3.3v | VDD |
| GND | GND |
| GPIO4 (RX) | TXD |
| GPI0 5(TX) | RXD |

** SPI Communication Protocall **

1. SPI stands for Serial Peripheral Interface, and it is a synchronous serial data protocol used by microcontrollers to communicate with one or more peripherals. For example, your ESP32 board communicating with a sensor that supports SPI or with another microcontroller.

i. HSPI Pins: (ESP32)

- MISO (Master In Slave Out) - GPIO 12

- MOSI (Master Out Slave In) - GPIO 13

- SCK (Serial Clock) - GPIO 14

- CS (Chip Select) - GPIO 15

ii. VSPI Pins: (ESP32)

- MISO (Master In Slave Out) - GPIO 19

- MOSI (Master Out Slave In) - GPIO 23

- SCK (Serial Clock) - GPIO 18

- CS (Chip Select) - GPIO 5

- LoRa (REYAX RYLR 890) - ESP32 :->

| ESP32 | LoRa (REYAX RYLR 890) |
| --- | --- |
| 3.3v | VDD |
| GND | GND |
| GPIO16 (RX) | TXD |
| GPI0 17(TX) | RXD |

** LoRa RYLR896 **

- AT Command :->

| Command | Description | Example |  |  |
| --- | --- | --- | --- | --- |
| AT |  |  |  |  |

Some documentation AT Command link -> https://www.cavliwireless.com/blog/nerdiest-of-things/an-introduction-to-cellular-at-commands.html

- AT

- AT+ADDRESS

- AT+NETWORKID

- AT+BAND

- AT+PARAMETER  -  AT + PARAMETER = 10,7,1,7

- Spreading Factor

- Bandwidth

- Coding Rate

- Programmed Preamble

- AT+SEND

5) Spreading Factor (SF) :

- SF controls how long the signal stays in the air and how much it spreads.

- Higher SF → Better range & sensitivity, but slower transmission.

- Lower SF → Faster transmission, but less range

| SF Value | Range | Speed |
| --- | --- | --- |
| SF7 | Short | Fast ⚡ |
| SF10 (Selected) | Medium | Medium |
| SF12 | Long | Slow |

Chat GPT -> https://chatgpt.com/c/67adf5d6-8334-8007-8bfc-cddfbaa176d6

LoRa Document : LoRa — LoRa documentation

3. How RF & Serial Parameters Work Together

- RF parameters (SF, Bandwidth, Coding Rate, Preamble) affect wireless communication between LoRa nodes.

- Serial parameters (Baud Rate, Parity, Data Bits, Stop Bits) affect UART communication between LoRa and ESP32.

- A balance is needed:

  - If SF is high, baud rate should be moderate (to avoid delays).

  - If Bandwidth is high, SF should be lower (to avoid noise).

  - If CR is high, baud rate should be low to avoid errors.

- AT+RESET

- AT+MODE=1

- AT+IPR=115200

- AT+PARAMETER=7,3,4,5

- AT+BAND=868500000

- AT+ADDRESS=120

- AT+NETWORKID=00001

- AT+CPIN=ANUJ9052985897

- AT+CRFOP=10

- AT+SEND=50,5,HELLO -> (send data )

** Micro SD Card Adaptor **

- Function Operation:

| FILE | write |  | FS | Open |  | MODE | FILE_WRITE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | getLastWrite |  |  | Exists |  |  | FILE_APEND |
|  | Read |  |  | Remove |  |  | FILE_READ |
|  | Peek |  |  | Rename |  |  |  |
|  | Flush |  |  | Mkdir |  |  |  |
|  | seek |  |  | rmdir |  |  |  |

- ESP8266 & Micro SD Card Adaptor:

- Install the library -> “#include <SPI.h>”, “#include <SD.h>”

- Connect with 5V with Micro SD Card Adoptor [External Power Supply]

- CODE ->

#include <SPI.h>

#include <SD.h>

if (!SD.begin(15)) {

Serial.println("Initialization failed!");

return;

}

| ESP8266 | Micro SD Card Adaptor |
| --- | --- |
| GND | GND |
| 5v -> external | VCC |
| D8 (GPIO 15) | CS |
| D5 (GPIO 14) | SCK |
| D7 (GPIO 13) | MOSI |
| D6 (GPIO 12) | MISO |

- ESP32 & Micro SD Card Adaptor:

- Install the library -> “#include <SPI.h>”, “#include <SD.h>”

- Connect with 5V with Micro SD Card Adoptor

- CODE ->

#include <SPI.h>

#include <SD.h>

#define CS_PIN 5

while (!SD.begin(CS_PIN))

{

Serial.println("Failed to Connect the SD Card...");

return;

}

| ESP32 | Micro SD Card Adaptor |
| --- | --- |
| GND | GND |
| 5v | VCC |
| GPIO 5 | CS |
| GPIO 18 | SCK |
| GPIO 23 | MOSI |
| GPIO 19 | MISO |

** Adafruit Fingerprint Sensor Library **

| R503 Fingerprint Module | ESP32 |
| --- | --- |
| Power Supply | 3V3 |
| GND | GND |
| TXD | RX (GPIO 16) |
| RXD | TX (GPIO 17) |
| WAKEUP | Not Connected |
| 3.3VT | 3.3 V |

Descrition:

The Adafruit Fingerprint Sensor Library is a software library designed to interface with fingerprint sensors (like the R30x series) using Arduino or compatible microcontrollers. It provides functions to:

- Enroll new fingerprints.

- Verify fingerprints against stored templates.

- Delete fingerprints.

- Manage the sensor's database (e.g., clear all fingerprints).

This library simplifies the process of integrating fingerprint-based authentication into your projects.

- begin(): Initializes the fingerprint sensor and establishes communication.

- getImage(): Captures a fingerprint image from the sensor.

- image2Tz(): Converts the captured fingerprint image into a template for processing.

- createModel(): Combines two fingerprint templates (from the same finger) to create a final model for storage.

- storeModel(): Stores the fingerprint template at a specific ID in the sensor's memory.

- fingerFastSearch(): Searches the sensor's database for a matching fingerprint.

- deleteModel(): Deletes a fingerprint template associated with a specific ID.

- emptyDatabase(): Clears all fingerprint templates from the sensor's memory.

** 4x4 Matrix Keypad **

| 4x4 Matrix Keypad | ESP32 |
| --- | --- |
| R1 | D13 |
| R2 | D12 |
| R3 | D14 |
| R4 | D27 |
| C1 | D26 |
| C2 | D25 |
| C3 | D33 |
| C4 | D32 |

YouTube Video:  https://www.youtube.com/watch?v=OTob_AeBUrw

** 16 Channel Analog Multiplaxer(CD74HC4067) **

- Connection With ESP8266:

| CD74HC4067 | ESP8266 |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| S0 | D1 (GPIO5) |
| S1 | D2 (GPIO4) |
| S2 | D3 (GPIO0) |
| S3 | D4 (GPIO2) |
| EN (Enable) | GND or GPIO12 |
| SIG | D5 (GPIO14) |
| C0 – C15 | LEDs (With Resistors) |

- Connection With ESP32:

| CD74HC4067 | ESP32 |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| S0 | GPIO 18 |
| S1 | GPIO 19 |
| S2 | GPIO 21 |
| S3 | GPIO 22 |
| EN (Enable) |  |
| SIG | GPIO 32 (for sensors - Analog input) |
| C0 – C15 | LEDs (With Resistors) |

** RFID-RC522 **

- Connection With ESP8266:

| RFID-RC522 | ESP8266 |
| --- | --- |
| SDA (SS) | GPIO 15 (D8) |
| SCK | GPIO 14 (D5) |
| MOSI | GPIO 13 (D7) |
| MISO | GPIO 12 (D6) |
| GND | GND |
| RST | GPIO 0 (D3) |
| 3.3 V | 3.3V |

- Connection With ESP32:

| RFID-RC522 | ESP32 |
| --- | --- |
| SDA (SS) | GPIO 5 |
| SCK | GPIO 18 |
| MOSI | GPIO 23 |
| MISO | GPIO 19 |
| GND | GND |
| RST | GPIO 4 |
| 3.3 V | 3.3V |

** SPI Support ESP32 Pin **

- ESP32 have Support two Type of Buses:

- VSPI (Default SPI Bus)

- HSPI (Secondary SPI Bus)

| SPI | MOSI | MISO | SCLK | CS |
| --- | --- | --- | --- | --- |
| VSPI | GPIO 23 | GPIO 19 | GPIO 18 | GPIO 5 |
| HSPI | GPIO 13 | GPIO 12 | GPIO 14 | GPIO 15 |

** SPI with Star Topology (Separate CS lines) **

In this setup:

- MOSI, MISO, and SCK lines are shared among all devices.

- But each device has its own separate CS (Chip Select) pin from the master.

- All Device of MISO Line have the same line with separate the 330 Ω Registor’s.

  - ESP32 -> 330 Ω -> MISO  (Every Devide Has its OWN Registor’s)

- In Some Cases Only Connect the 330 Ω Registor on SD Card Adoptor Module

** UART Communication Protocall **

- UART for ESP32

  - There are 3 UART Support -> UART0, UART1, UART2

  - UART0: this UART is used the basic serial communication. Like, Printing in the terminal.

  - UART1: this UART is used for the connecting to other Device support. Like, Fingerprint Sensor and the Pin is -> pair -> RX, TX -> 32, 33 | 25, 26 | 27, 14 | etc.

  - UART2: this UART is used for the connecting to other Device support. Like, GPS Module and the Pin is -> pair -> RX, TX -> 16, 17|