*** Raspberry_pi Coding ***

- Boot raspberry_pi (ubuntu server):

- Select one SD Card (32GB, 64GB)

- Format the SD Card

- Open raspberry_pi Imager (Download from Oficial Website)

- Insert SD Card in Your Computer by using OTG (USB)

- Install Ubuntu 22.04 LTS (Server) 64 bit

- Configur SSL Setting by wireless LAN :

- ID -> ubuntu

- Pass -> 1234

- HostName -> ssh ubuntu@raspberrypi.local

- Wifi:

- SSID:-> Airtel_jyot_6722

- Pass:-> air97311

- Now Install Ubuntu Server

- Setup Yacto Project:  -> (imp)

- Step – 1:

- Create Directory “Workspace/yacto_project”  -> choose any name

- Clone Repository -> “Workspace/yacto_project”

- git clone https://github.com/yoctoproject/poky.git -b krikstone

- git clone https://github.com/agherzan/meta-raspberrypi.git -b krikstone

- Create Folder -> “Workspace/yacto_project”

- Folder name -> sources

- Step – 2:

- Update local.conf: (/Workspace/yacto_project/poky/build/conf/local.conf)

- path -> Workspace/yacto_project/poky:

- source oe-init-build-env

- Top:

- MACHINE ??= "raspberrypi4"

- Middle:

- SOURCES = "/home/ubuntu/Workspace/yacto_project/sources"

- DL_DIR ?= "${SOURCES}/downloads"

- SSTATE_DIR ?= "${SOURCES}/sstate-cache"

- TMPDIR = "${SOURCES}/tmp"

- Bottom:

- CONF_VERSION = "2"

- RM_OLD_IMAGE = "1"

- INHERIT += "rm_work"

- Step – 3:

- Update bblayers.conf: (Workspace/yacto_project/poky/build/conf/bblayers.conf)

- # POKY_BBLAYERS_CONF_VERSION is increased each time build/conf/bblayers.conf

- # changes incompatibly

- POKY_BBLAYERS_CONF_VERSION = "2"

- BBPATH = "${TOPDIR}"

- BBFILES ?= ""

- BBLAYERS ?= " \

- /home/ubuntu/Workspace/yacto_project/poky/meta \

- /home/ubuntu/Workspace/yacto_project/poky/meta-poky \

- /home/ubuntu/Workspace/yacto_project/poky/meta-yocto-bsp \

- /home/ubuntu/Workspace/yacto_project/meta-raspberrypi \

- "

- Step – 4: Run Command

- Workspace/yacto_project/poky:

- source oe-init-build-env

- Workspace/yacto_project/poky/build:

- bitbake core-image-full-cmdline

1.2: For server Ubuntu installetion: (WiFi)

- ID -> ubuntu

- Pass -> 1234

- HostName -> ssh ubuntu@raspberrypi.local

1.3: For server Ubuntu installetion: (LAN)

- ID -> jellyfish

- Pass -> 1234

- HostName -> ssh jellyfish@raspberrypi.local

1.3: WiFi:

- SSID:-> Airtel_jyot_6722

- Pass:-> air97311

*** Project Guidline for Development ***

- Install the Library:

- GPIO:  sudo pip3 install RPi.GPIO

- pip3 install vpython

Quick-Reference Notes — Yocto, Poky & Layers (Simple English)

1. Yocto Project (the framework)

- Open-source toolkit to build custom Linux for any embedded device.

- Gives you the build engine (BitBake), documentation, and a standard layer system.

- Does not ship as an OS itself; it only builds one.

2. Poky (the starter pack)

- The official “reference distribution” that comes with Yocto.

- A folder you clone: it bundles

  - BitBake (the build engine)

  - Core layers: meta/, meta-poky/, meta-yocto-bsp/.

- Purpose: a ready-made starting point so you don’t begin from scratch.

- Used only at build time; Poky files are not copied into the final image.

3. Layer (the building block)

- A directory that holds recipes + config for one topic (e.g., Python, Qt, your app).

- Think “plugin”: add or remove layers to change what goes into the image.

- Typical structure:

bash

CopyEdit

meta-myproject/

└─ conf/layer.conf

└─ recipes-*/…*.bb

4. Relationship

- Poky contains several layers, plus BitBake, and sets default policies.

- Your custom layers sit alongside Poky’s layers in the build.

- BitBake reads all active layers → combines recipes → creates the final rootfs, kernel, etc.

5. Best Practice

- Clone Poky but do not edit its layers.

- Create your own layer for every project feature:

bash

CopyEdit

source poky/oe-init-build-env

bitbake-layers create-layer ../meta-myproject

bitbake-layers add-layer    ../meta-myproject

- Put all custom recipes, patches, configs inside meta-myproject/.

- When Yocto/Poky updates, you can pull the new version without merge pain.

6. Build-vs-Run Reminder

| Phase | What’s used | Ends up in device? |
| --- | --- | --- |
| Build | Poky layers + your layers + BitBake | No (tools only) |
| Runtime | Packages defined in layers | Yes (your app, drivers, libs) |

7. Handy Commands

- Set up env: source oe-init-build-env

- List layers: bitbake-layers show-layers

- Build minimal image: bitbake core-image-minimal

- Clean layer build: bitbake -c cleanall <recipe>

8. One-Sentence Memory Hooks

- Yocto is the workshop, Poky is the starter toolkit, Layers are the parts.

- Edit layers, never Poky itself.

- Poky helps build; it doesn’t live inside the final OS.